Description of the Evaluationboard for the Atmel Microcontrollers AT89Sxx and (A)T89C51xx

The following figure shows the component side of the Evaluationboard.

The Evaluationboard is a 4 layer PCB with 100mm x 100mm size equipped with two PLCC44

sockets to accept the following microcontrollers with internal Flash Memory.

Supported Microcontrollers:

AT89S51J

AT89S52J

AT89S53J

AT89S8252J

AT89C51RB2J

AT89C51RC2J

T89C51RD2J

T89C51IC2J

T89C51AC2J

Warning:

The micro controller shall be installed on one of the sockets U1 and U2 (T89C51AC2 only) depending on the

type selected. Simultaneous operation of two micro controllers on socket U1 and socket U2 isn't permissible

and will lead to malfunction !

All integrated circuits are socket mounted to allow easy replacement.

The board contains no surface mounted devices to allow soldering by not well experienced people.

This will ensure easy replacement of parts and adaptation to the application specific requirements.

The board comes with a 96pin connector DIN41612-C (P1) and a 9pin DSUB female connector (J1) for the

RS232 interface. All microprocessor I/O ports and all of the board I/O signals are connected to the connector P1.

A 10 pole pin header (X1) compatible to the Atmel ATISP Programmer Cable allows easy in-circuit programming

of the FLASH memory of the AT89Sxx microcontroller series.

(refer to Atmel Application Note AT89ISP_Cable_doc3310.pdf)

The in-circuit programming  of the (A)T89C51xx micro controller series is made via the RS232 interface by using

the on-chip Boot ROM of the micro controller (refer to Atmel documentation)

The crystal for generation of the micro controller clock is also socket mounted (precision single pin sockets).

This allows easy clock frequency adaptation to the different applications and the micro controller used.

Interfaces:

The Evaluation Board provides the following interfaces:

- max. 32 + 2 programmable I/O lines depending on the used micro controller

- RS232 interfaces (port P3.1 and P3.0) available at P1 and J1

- RS422/485 interface available at P1

- SPI interface depending on the used micro controller

- I2C interface (T89C51IC2 only)

- micro controller on-chip (8 channel 10 bit) analog to digital converter (T89C51AC2 only)

- 8 channel 10 bit or 8 channel 10 bit analog to digital converter (U11; optional)

The serial interfaces are user selectable by jumpers on pin header X2.

Communication control of the serial interfaces may be provided by jumpers on pin header X2 using the

micro controller ports P1.2, P1.3 and P1.4

Memory:

- On-chip memory (256 bytes SRAM, up to 64K FLASH, up to 2K EEPROM, up to 2K XRAM) depending on

the used micro controller.

- optional 256 or 1024 bit serial EEPROM (U7) for nonvolatile storage of parameters and setting information

- optional 8K, 32K, 128K SRAM or 128K, 256K, 512K FLASH EPROM/ EEPROM on socket U9 as data storage

The memory type (socket U9) selection is made via jumpers on the pin header X3.

Access to the SRAM, the FLASH EPROM or EEPROM on socket U9 is made via the XDATA space of the micro controller.

In this case the port P0 is used as databus. Port P2 delivers the higher address byte while the lower address byte is provided

by the address latch U6.

Three user selectable I/O lines of the micro controller may be used for bank switching (addresses A16, A17 and A18).

The four I/O lines of the serial EEPROM can be connected to any of the I/O ports of the micro controller by means

of external connections on P1.

If socket U9 is equipped with SRAM a battery backup feature is provided. The backup battery shall be connected to

P1 pins c1 and c2. Switch-over between Vcc (+5V) and the battery is performed by the reset circuit U8.

The reset circuit U8 also contains a watchdog and a power fail monitor. The signals to control the watchdog (signal WDI)

and to  read the power fail monitor (signal PFO/) may be connected to any of the I/O ports of the micro controller by means

of external connections on P1.

Power Supply:

The Evaluationboard is powered by a single 5V DC laboratory power supply.

Function of the jumper locations at pin header X2:

jumper 1-2 selects RS232 interface

jumper 3-4 selects RS422/485 interface

jumper 5-6 connects the CTS input signal of the RS232 interface to port P1.3

jumper 7-8 connects the CTS input signal of the RS232 interface to port P1.4

jumper 8-10 connects the RTS output signal of the RS232 interface to port P1.2

jumper 9-10 connects the signal TX_ENA_RS422 for transmission control of the RS422/485 interface to port P1.2

jumper 7-9 connects the signal TX_ENA_RS422 for transmission control of the RS422/485 interface to port P1.4

jumper 11-12 disables the RS422/485 line receiver during transmission on the RS485 interface

jumper 13-14 connects a termination (R3) on the RS422 input lines

Remark:

For RS485 interface connect P1 pins b21 with b19 and P1 pins b20 with b18.

Transmission control (data direction) of the RS485 interface is user selectable via jumpers on X2 as well as by means

of an external connection of P1 pin b17 to any of the I/O lines of the micro controller.

Function of the jumper locations at pin header X3:

The required jumper setting for the different memory types are:

SRAM 8KBytes; DIL28 case (e.g. Hitachi HM6264, Epson SRM2064, Toshiba TC5564, Toshiba TC5565)

4-5 (with battery backup); 5-6 (without battery backup)

11-12

14-15

17-18

20-21

SRAM 32KBytes; DIL28 case with /CS; /OE; /WE (e.g. Hitachi 62256, Epson 20256)

2-3

4-5 (with battery backup); 5-6 (without battery backup)

11-12

14-15

16-17

20-21

SRAM 32KBytes; DIL28 case with /CS1; CS2; /WE (e.g. Toshiba TC55257, Epson 20257)

2-3

4-5 (with battery backup); 5-6 (without battery backup)

11-12

14-15

16-17

19-20

SRAM 128KBytes; DIL32 case (e.g. Epson SRM20100)

2-3

4-5 (with battery backup); 5-6 (without battery backup)

7-8

11-12

14-15

16-17

20-21

FLASH EPROM or EEPROM 128KBytes; DIL32 case  (e.g. Intel i28F010, AMD AM29F010, Atmel AT28C010)

1-2

5-6

8-9

13-14

16-17

20-21

FLASH EPROM or EEPROM 256KBytes; DIL32 case  (e.g. Intel i28F020, AMD AM29F020)

1-2

5-6

8-9

10-11

13-14

16-17

20-21

FLASH EPROM or EEPROM 512KBytes; DIL32 case  (e.g. Intel i28F040, AMD AM29F040, Atmel AT28C040, Atmel 49F040)

1-2

5-6

8-9

10-11

13-14

16-17

20-21

Pinning of connector P1:

Pin	Signal	Pin	Signal	Pin	Signal

1a	+5V	1b	+5V	1c	VBATT

2a	GND	2b	GND	2c	GND

3a	A0	3b	P0.0 (AD0)	3c	P1.0 (T2, XTALB1, AN0)

4a	A1	4b	P0.1 (AD1)	4c	P1.1 (T2EX/, SS/, AN1)

5a	A2	5b	P0.2 (AD2)	5c	P1.2 (ECI, AN2)

6a	A3	6b	P0.3 (AD3)	6c	P1.3 (CEX0, AN3)

7a	A4	7b	P0.4 (AD4)	7c	P1.4 (SS/, CEX1, AN4)

8a	A5	8b	P0.5 (AD5)	8c	P1.5 (MOSI, CEX2/MISO, AN5)

9a	A6	9b	P0.6 (AD6)	9c	P1.6 (MISO, CEX3/SCK, AN6)

10a	A7	10b	P0.7 (AD7)	10c	P1.7 (SCK, CEX4/MOSI, AN7)

11a	P2.0 (A8)	11b	PSEN/	11c	P3.0 (RXD)

12a	P2.1 (A9)	12b	ALE	12c	P3.1 (TXD)

13a	P2.2 (A10)	13b	RESET	13c	P3.2 (INT0/)

14a	P2.3 (A11)	14b	RESET/	14c	P3.3 (INT1/)

15a	P2.4 (A12)	15b	EXT_RESET	15c	P3.4 (T0)

16a	P2.5 (A13)	16b	WDI	16c	P3.5 (T1)

17a	P2.6 (A14)	17b	TX_ENA_RS422	17c	P3.6 (WR/)

18a	P2.7 (A15)	18b	TX_RS422_T	18c	P3.7 (RD/)

19a	A16	19b	TX_RS422_C	19c	NC1 (XTALB2, AGND)

20a	A17	20b	RX_RS422_T	20c	NC2 (PI2/SDA, P4.0)

21a	A18	21b	RX_RS422_C	21c	NC3 (VAREF)

22a	U9_CS/	22b	RTN_RS422	22c	NC4 (PI2/SCL, P4.1)

23a	GND	23b	93Cx6_SK	23c	ADC_SCLK

24a	TX_RS232	24b	93Cx6_CS	24c	ADC_CS/

25a	RX_RS232	25b	93Cx6_DO	25c	ADC_DOUT

26a	n.c.	26b	93Cx6_DI	26c	ADC_DIN

27a	ADC_AGND	27b	ADC_VREF	27c	ADC_SSTRB

28a	ADC_CH0	28b	ADC_CH4	28c	ADCSHDN/

29a	ADC_CH1	29b	ADC_CH5	29c	n.c.

30a	ADC_CH2	30b	ADC_CH6	30c	PFI

31a	ADC_CH3	31b	ADC_CH7	31c	PFO/

32a	GND	32b	GND	32c	GND

Pinning of connector J1 (RS232 interface):

Pin	Signal

1	not connected

2	TX_RS232

3	RX_232

4	not connected

5	Ground

6	not connected

7	CTS/RX-RDY

8	RTS/TX-RDY

9	not connected

This signal assignment allows connection to a PC COMx interface using a standard 1:1 serial cable.

Warranty:

The provided information is as provided without any warranty for function and correctness.

License:

The limited re-production of the PCB is under license. After receiving the license fee of 25 Euro or 25 US$ the

complete board file (EvaluationBoard AT89xx.brd) will be provided. The schematic and other informations are free.

The design of the EvaluationBoard had been made using the EAGLE Version 4.09 PCB Design Tool.

On your interest please contact:

Reinhard Vogg

Kiefernweg 9

D-87448 Waltenhofen/Germany

Tel.: +49/831/12424 or +49/831/5127076

E-Mail: Vogg.Reinhard@VDI.de	Last update 17.8.2003