Notes:

Filters.sch (and Filters.brd) contains general purposes filter and amplifier circuits. Each circuit is also available as library module in abi.lib.

LOW PASS FILTER: It is Sellen-Key architecture, 2/4 pole low-pass filter circuit. For multiple stages a library module is also provided (LPF01). The exact resistor values of the low-pass filters can be calculated with the program “resistor.exe” (Home made and DOS-based, sorry!), which is also provided. It calculates closest resistor value from 2 standard 5% resistors. Low pass filter design programs (free!) are available from the web sites www.microchip.com or www.ti.com.

NOTCH FILTERS: There are two notch filter circuits provided. Circuits based on the application notes from National Instruments and Texas Instruments web sites respectively. Notch filter with UAF42 is from Burr Brown AN. Notch frequency is 60Hz. Library module is also available. (notch42)

UNIVERSAL AMPLIFIER: Simple DC gain amplifier with gain and offset adjustment. For a fixed gain also a separate Rf (feedback resistor) is on the circuit.

If you need help: send e-mail to: yildirim@uab.edu

Abi - Enjoy!!!!