SMD 4 Layer design with EAGLE

16-Aug-1999

1. Set the defaults in AUTO route to the following:

Grid Size:	.0125

Trace Width:	.0080

Via Size:	.0240

Drill Size:	.0160

Make sure that the defaults are fixed in the job. You may have to done a dummy route, stop and ripup to have the values saved for the board.

2. Set layer 2 to GND (“layer 2 $GND”). Set layer 15 to VCC (“layer 15 $VCC”)

3. Set AUTO route so that all route optimization is turned off.

4. AUTO route VCC and GND (“auto VCC GND;”) This will punch straight through to the power planes.

5. Set AUTO route optimization back on.

6. AUTO route all signals.

7. If there are remaining un-routed signals rename the VCC layer back to Route15

(“layer 15 Route15”) and AUTO route the remaining.

8. Poly and pour the VCC and GND layers.

Fred Collins

fcollins@bigfoot.com